## Simulating self motion II:

## A virtual reality tricycle

R. Allison1, L. R. Harris2,3, A. Hogue1, U. Jasiobedzka1, H. Jenkin, M. Jenkin1,

J. Laurence, G. Pentile1, F. Redlick3, J. Zacher, D. Zikovitz3

Centre for Vision Research,

and

Departments of Computer Science1, Psychology2 and Biology3

York University,

4700 Keele St., Toronto, Ontario, Canada, M3J 1P3

Contact author: M. Jenkin – jenkin@cs.yorku.ca

### Abstract

The previous paper concluded that when simulating self motion, virtual reality designers ignore non-visual cues at their peril. Providing such cues presents significant challenges. One solution is to accompany  visual displays with corresponding real physical motion. This stimulates the non-visual sensory systems in the natural way: but allowing real movement requires space. Technologies such as head mounted displays and CAVETMs can be used to provide large immersive visual displays within small physical spaces. It is difficult, however, to provide virtual environments which are as large physically as they are visually. A fundamental problem is that tracking technologies which work well in a small, enclosed environment do not function well over longer distances.  Here we describe Trike -- a ‘rideable' computer system that can be used to present large virtual spaces both visually and physically and thus provide appropriately matched stimulation to visual and a number of non-visualsensory systems.

# 1. Introduction

Although tracking technologies have seen considerable advances, fundamental problems remain, especially for large physical footprints. With few exceptions, existing tracking systems are designed around technologies that limit the user to a small physical footprint. For example, mechanical tracking systems such as Fake Space's BOOM, and Puppetworks’ trackers, physically connect the wearer to a fixed position and thus the user is mechanically tethered to a point in space. Wireless electromagnetic systems such as Ascension Technology's Flock of Birds tracking system and Polhemus’ Fastrak remove the mechanical link, but have a limited operational range. For example the Flock of Birds has an operational range of 4 feet with the standard transmitter and 10 feet with the extended range unit.  Vision-based trackers such as Northern Digital's POLARIS have the potential of a longer operating range but their accuracy degrades with distance due to the use of triangulation to measure position. Acoustic tracking systems are limited by the decay in the audio signal as a function of distance and the sound absorption properties of air.  Existing virtual reality systems may provide a large visual space but the bulk of existing tracking technologies limit the user to moving in a relatively small physical world.

Various alternatives have been proposed to extend the range over which an immersive VR display can operate. Perhaps the most extreme version of these has been the development of wearable computers (e.g. [1,2,3]). These systems provide a computer and interface that is ‘worn' by the user and that augments their normal senses as they move. Although most existing wearable systems are designed to provide two-dimensional visual overlays on the wearer's normal visual field some wearable computer systems provide more immersive visual displays.  For example, [4] describes a wearable computer system that in part mimics a pair of binoculars but with identical  views in both eyes.  The wearer looks through the ‘binoculars' and is presented with a computer-enhanced version of the environment. Feiner [5] describes a backpack-based system in which the user views the world through a head-mounted display in which synthetic elements can be superimposed over the normal view.  But the lack of a static base station and some mechanism for measuring displacement relative to an earth-fixed point make estimating absolute position very difficult in this system.  Only relative orientation information is available for the generation of the visual display.

Technologies which track over shorter ranges can be patchworked together to form a mosaic of trackable regions.   For example, a network of Flock of Bird transmitters can be used to provide coverage over a large space but at significant cost and the system still requires a connection from the receiver to the base station. One successful application of a patchwork approach is Northern Digital’s OPTOTRAK system. This system senses active infrared markers placed on the subject with a number of camera trackers to provide long-range tracking. Even with an unlimited budget, this type of tracking technology will only provide tracking over a physical space the size of the environment in which the tracker is installed.

Although the Global Positioning System (GPS) -- especially in its differential mode -- could theoretically be used to provide tracking information over larger ranges, it is not without its problems.  For example, it cannot be used in environments where no clear line of sight exists to the satellites such as indoors, near mountain ranges, in forests, and built up urban areas.  It also has a relatively slow update rate (see [6] for a description of GPS and its limitations.)  That being said, in appropriate outdoor environments, GPS can be used to correct drift in other sensors (see below).

# 2. Simulating medium- and long-range motion

In order to provide a virtual environment over a large physical space and to permit the experiments described in the previous paper, we have developed a rideable computer system (Trike) based on a commercially available tricycle, which can be used to provide a full range of naturalistic cues while users work within a large scale virtual environment.  Standard virtual reality technology is used to provide an immersive visual display relative to the Trike itself, the effort of pedalling Trike around provides cognitive and proprioceptive cues consistent with normal navigation and the physical motion of Trike generates the normal spectrum of non-visual cues .  Trike is instrumented so that its motion relative to some initial base frame can be computed, and can be tracked using a ground-based visual system in order to deal with drift from the vehicle-based tracking system.

In addition to providing a natural mechanism for navigation in a large-scale virtual world, Trike can also be used as a powerful research tool for investigating the relative importance of different visual and non-visual cues to the perception of self-motion in virtual environments (see previous paper).

The complexity of the perceptual cues that an operator experiences when they move  (ref: previous paper) presents a challenge to those attempting to simulate long-distance motion without actually moving the operator through the corresponding distance. A number of technologies have been developed and deployed in order to try to simulate these movements by stimulating one or more non-visual sensory systems while ignoring the others. Here we review some of the technologies employed to simulate medium- and long-range motions.

Static Automotive/Aircraft Simulators.

Static (non-moving) simulators have found a wide range of applications in everything from aircraft pilot training to training student drivers.  The devices replicate the appearance and functionality of that part of the device needed for the training to be performed. The device is static, but the visual displays and instruments are driven by a computer-based simulation of the vehicle’s systems to provide realistic responses to operator inputs. The sophistication of the simulation and displays runs the range from accurate, large aircraft simulations, such as those manufactured by Atlantis Aerospace, to a simple PC with a steering wheel and brake and accelerator pedals.

The sensation of motion in these devices is generated primarily by visual cues, and the expectation of motion from the state of the instruments and controls.

Virtual Reality Bicycles.

A number of laboratories have adapted stationary bicycles  (see [7]) – typically a road bicycle mounted on a trainer stand -- to simulate long-range motion. The bicycle is mounted on a stand, and the steering angle and drive wheel are instrumented. Coupled with an appropriate kinematic model, this permits the simulation of motion through large-scale space (see [8] for one approach to constructing a Virtual Reality Bicycle.)  It is even possible to provide computer control of the resistance that the operator experiences when cycling. For example the Computrainer system by RacerMate Inc., incorporates a computer-controlled device that opposes motion of the pedals.  This can then realistically simulate the resistance associated with pedalling uphill.

A virtual reality bicycle simulates many of the non-visual cues associated with long-range motion, however,  as the bicycle does not physically move relative to the ground, neither the rotation sensing or translation sensing parts of the vestibular system are activated by the motion of the bicycle itself. It is also important to note that virtual reality bicycles are usually fixed in place and do not lean like real bicycles. Tilting a static bicycle to the side as it goes round a corner is possible; a low-slung version has been developed by Heinrich Büelthoff and his group (see [7]). Tilting the bicycle makes the steering feel more natural but does not by any means simulate all the physical forces (eg. Centripetal forces and linear and angular accelerations) that a cyclist normally experiences when turning.

Virtual Reality Hang-glider.

Virtual reality hang-gliders attempt to provide a realistic way to control a hang-glider. The Dreamglider system marketed by Dreamality Technologies is typical. The operator is strapped into a harness that mimics the harness normally worn by a hang-glider operator and either views a fixed monitor or wears a head-mounted display. Similar to flying a real hang glider, the operator shifts their weight in the harness to control the flight of the device. NTT’s Virtual Hang-glider adds simulated wind to the experience. Although these devicesmay be great toys, they cannot hope to present the physical motions associated with real hang-gliders.

Virtual Reality Treadmills.

Standard exercise treadmills can be instrumented to permit an operator to walk in one dimension [9]. The treadmill can either be electrically controlled to center the operator within its operational range or a non-motorized treadmill can be used with the subject walking to provide treadmill motion.  This second option is not as desirable as this requires rather unnatural motion on behalf of the operator.  An obvious restriction is that only one-dimensional motion is possible. To address this issue, two-dimensional treadmills such as the Omni-Directional Treadmill (ODT) manufactured by Virtual Space Devices and the Torus Treadmill at the University of Tskuba have been developed (see [10] and [9]). An individual walking on the surface of the ODT can move in any direction, and the device maintains the user’s position near the device’s center.  The ODT is essentially two continuous belts, one for the X direction, one for the Y. The X belt is supported by the Y belt and is mechanically transparent so that the Y motion is conducted up through the X belt. Each belt is made up of series of rollers woven together. Each belt is motorized and motion of the operator relative to the treadmill is tracked. The belts are rotated so as to maintain the operator within the physical extent of the device.

Although the ODT can provide many non-visual cues to self motion, the limited physical extent of the device requires that the device has to keep moving the operator back to the centre of its operating range.  For relatively short-duration motions, or motions followed by stationary periods, these recovery motions can be of short duration and very low magnitude in an attempt to keep them below the operator’s perceptual threshold. For continuous motion however, these re-centering motions are likely to confound the normal non-visual sensations experienced by the operator.

Physical Motion Bases.

Physical motion bases have been used for aircraft pilot training since in 1920’s. One of the earliest successful physical simulators is the Link trainer. The Link trainer originally provided the operator with a control column and control wheel, two foot pedals, and various flight and navigation instruments. The device sat on four pneumatic bellows and used air pressure to simulate flight motion such as climbing, diving and banking. The device responded to commands from the control column, and through a series of linkages used the air to adjust the pose of the simulated cockpit.

Modern aircraft simulators such as those manufactured by CAE or TDI, utilize 6 DOF motion bases coupled with complex visual displays and simulation software. The physical motion base typically has a very restricted motion envelope, which means that large physical motions must be simulated through small-range physical motions.

For translation, motion bases can provide the initial linear acceleration and then, as for the 2D treadmill, attempt to return users to the centre of the operating range at subliminal accelerations (washout) during periods of constant velocity or no movement. They can also provide tilt to mimic an aircraft banking, or to simulate linear accelerations by directing an appropriate component of gravity (controlled by the amount of tilting) along the direction that a linear acceleration is expected.

Summary

These ingenious systems provide some of the natural outgoing cues, such as those generated by pedalling and steering (bicycle), walking (treadmill) or body orientation changes (hang gliding). However, with the exception of motion bases, they do not provide or simulate the non-visual sensory cues that would naturally be associated with the motion of their mobile counterparts.

3. Building a rideable computer

| Side View | Front View |
| --- | --- |

Figure 1. The Virtual Reality Tricycle (Trike). The Trike is based on an adult-sized tricycle which has been instrumented to measure the rotation of one of the rear drive shaft and the steering angle. A Polhemus Tracker provides a measurement of the rider’s head relative to the Trike. Visual display is presented via a V8 binocular HMD. Display is generated onboard via a SGI O2. Power, and optionally network access, is provided via a tether.

As our goal was to allow physical motion over a large area, it is important that Trike be tethered mo more than necessary to a base station. Computing and tracking is therefore performed on board the vehicle. The device is tethered only for power and optionally networking.  These could be removed with self-carried batteries and a wireless network link. As Trike has a relatively high payload capacity and can be tethered for power, weight and power consumption are not significant constraints on Trike's design. This permits standard off-the-shelf components to be used on the vehicle.  Trike (see Figure 1) is based around a standard size adult tricycle.  The drive train of the stock vehicle has been modified so that there is only a single gear and so that pedaling backwards causes the vehicle to go backwards.  The slack in the drive chain which is normally present in a bicycle or tricycle has been reduced in order to reduce backlash.

In order to estimate the vehicle's motion and position relative to some initial frame, the vehicle has been instrumented with a potentiometer connected to the steering axis that senses the steering direction and the drive wheel has been instrumented with an optical encoder so that its rotation can be measured. A small onboard microprocessor monitors these sensors as well as two buttons mounted on the handlebars. The microprocessor communicates with the main onboard computer via a standard serial link.

Figure 2. Bicycle/tricycle kinematics. The shaded ovals represent the three wheels of Trike. The instantaneous centre of curvature (ICC) must lie at the intersection of lines orthogonal to, and passing through the rotation axis of each wheel.

The main onboard computer is an SGI O2 with dual display (the ``two-headed option''). The two video outputs of the machine are fed to a binocular head-mounted display which is equipped with a Polhemus head tracker. The tracker's reference transmitter is mounted on the bicycle. The Polhemus tracker therefore reports the position of the helmet relative to Trike and Trike's instrumentation yields motion and direction information of the machine relative to space.  Thus the rider's position can be calculated from the combination of these two tracking systems and the appropriate visual display generated.

3.1 Tricycle Kinematics

In order to update the rider's position with respect to the world it is necessary to construct a kinematic model of the vehicle. Fortunately bicycles and tricycles have a straightforward kinematic model. Full details of the kinematic models of these and other wheeled vehicles can be found in [6] but the basic concept is sketched here.

Tricycles are steered vehicles in which changes in wheel orientation or steering direction are used to change the trajectory of the vehicle. For a wheeled vehicle to move without slippage each wheel must follow a circular course around the vehicle's instantaneous centre of curvature (ICC) and must roll on the ground with a velocity which is consistent with the geometry of the wheel placement. As power is provided to the wheels, the entire vehicle will then rotate about the ICC. The task of determining the kinematics of any wheeled vehicle, and the tricycle in particular, reduces to the task of determining the ICC of the vehicle.

The ICC must lie at the intersection of lines drawn through and perpendicular to the rotational axis of each wheel (see Figure 2).  Thus for a tricycle, the ICC must lie on a line passing through the rotational axis of the rear wheels which is perpendicular to the body of the tricycle.  The front wheel can be steered and thus the ICC lies on that part of the line passing through the rear wheels which intersects the line drawn along the axis of the front wheel.  Given a maximum steering angle of the front wheel, a tricycle has a minimum turning radius and rotates about a point on the line passing through the rear axle.

If the steered front wheel is set at an angle α from the straight-ahead direction and moves with ground contact velocity v, the tricycle will rotate with angular velocity w about a point lying a distance R along the line perpendicular to and passing through the rear wheels, where R and w are given by:

R=dtan(p2−α)       w=v(d2+R2)12

and d is the distance from the front to the rear axle as shown in Figure 2.

| User display | User display with interface overlaid |
| --- | --- |

Figure 3. A sample rideable environment. The environment is generated based on the current state of the head tracker and the Trike kinematic model (see text). The user interface can be overlaid over the rendered 3D environment. User interaction is via three buttons which can be used to manipulate existing objects in the environment and to initiate the creation of new objects.  The ‘brick’ pattern in the middle of the image is the currently selected texture for texturing surfaces. The interface permits both a users’ eye view as well as the birds’ eye view shown here.

Suppose that the tricycle is at some position  (x,y) and facing  along a line making an angle q with the x-axis at time t, i.e. it has pose xyqT. Then the ICC is given by:

ICC=(x−Rsin(q),y+Rcos(q))

and after a short interval Dt  the pose of the trike is given by

x'y'q'=cos(wDt)−sin(wDt)0sin(wDt)cos(wDt)0001x−ICCxy−ICCyq+ICCxICCywDt

This describes the motion of a bicycle rotating at distance R about its ICC with an angular velocity given by w.

The actual implementation on the Trike is complicated by the fact that the wheen encoder is mounted on one of the rear wheels, however the basic concept is as described here.

3.2 Constructing a visual world

Given the kinematic model of Trike and the state of the head tracker, the next step is  to generate a visual world in which to ride. Figure 3 shows a sample virtual environment

Used with the Trike. The environment was constructed using a point and click interface (described below) and consists of polygonal structures which can be texture mapped. The entire environment is  shaded and the operator's view is updated based on the state of Trike and the operator's head position and orientation.

Test environments are constructed through the 3D graphical interface shown in Figure 3.  The user interface was designed to allow the user to interactively create a virtual environment encompassing all three spatial dimensions. It allows the creation of simple geometrical objects such as boxes and rectangles through a point and click mechanism. Objects can also be imported following a VRML-like file format. The interface allows a user to specify operations such as move, rotate, scale, and shear to act upon existing objects. For many operations the construction interface manages the parameters necessary to allow such implicitly 3D operations to be performed as  sequences of 2D tasks. For example, an object may be moved along the XY, YZ, or XZ planes according to the settings on the interface. The interface also allows setting object properties such as color, transparency, and specularity. Operations through the interface can also take the form of assigning and manipulating object textures. Textures can be set on polygonal faces and then scaled, moved or rotated.

# 4. Extending the Trike’s operational range

Although the Trike can be used ‘as is’ over short distances, or for simulations over longer distances, the internal kinematic modeling of the device is not sufficiently accurate for the device to be used for medium- or long- range experiments into human performance in tasks such as cognitive mapping.  To study cognitive mapping, it is necessary for the Trike’s internal model to be accurate with respect to the external physical world. This is necessary so that the experimenters know the non-visual stimuli that the rider is experiencing and also to calibrate the path with respect to a possibly restricted workplace such as an arena to maintain the Trike within the boundaries of the real physical space..

| A. Trike with the addition of the visual target (large blue box). | B. Stereo tracking system |
| --- | --- |

Figure 4. Hardware to permit long-range tracking of the trike. A The trike is enhanced through the addition of a large visual target which is colour coded to aid tracking. B. Tracking is performed using a stereo camera system.

Inaccuracies in the Trike’s internal model arise due to the fact that its kinematic model is based only on internal sensors. Small errors in these sensors and in the kinematic model of the vehicle build up over time resulting, eventually, in a complete disassociation between the vehicles true physical position and its internal model.  This is related to the problem of pose maintenance in mobile robotics (see [6]), in which the ongoing pose of a mobile robot cannot be estimated using internal sensors only, and some reference to an external frame of reference is required in order to maintain the robot’s pose. For Trike, the internal model of the vehicle’s pose is inaccurate due to a number of factors including:

Wheel Slippage.

Figure 5: HSV coded view from each of the stereo cameras. The visual target is easily identified in the images as a dark box. The left eye’s image is shown on the left and the right eye’s image on the right so these images can be fused by uncrossed fusion.

The Trike’s sensor measures rotation of the vehicle’s wheel. As the wheel is not in perfect contact with the ground, and as the wheel deforms as it rolls, the motion of the wheel does not correspond exactly to the motion of the vehicle on the ground.

### Steering Angle Error.

### The potentiometer, which is used to measure the angle of the steering wheel is calibrated against two hard stops. The orientation of these two hard stops is measured by hand and this can introduce error. Although the steering angle can be varied continuously and the resulting resistance of the potentiometer changes continuously, this value is then discretized and assumed to change in direct proportion with the steering angle.

Kinematic Modelling Error.

The kinematic model of the Trike described above is quite simple. It assumes perfect wheel-ground contact, idealized wheels, etc. It is unlikely that these assumptions will hold in practice.

The accumulation of these errors makes the internal model of the Trike drift over time far from what is observed in the real world.  If the Trike is to be used in experiments which involve registering the position of the Trike with landmarks in the world, a mechanism must be provided to correct the model whenever possible to diminish, or at least bound, these errors.

Figure 6. Stereo view of the Trike showing the tracked region marked by a box with a dot in the middle. The left eye’s image is shown on the left and the right eye’s image on the right so these images can be fused by uncrossed fusion.

To address these issues, a stereo vision system has been developed which is able to track the Trike as it moves through the world. This stereo system obtains independent estimates of the vehicle’s position in a global coordinate system, and sends the proper world coordinates through a network connection to the Trike, where they can be integrated with the internal pose estimate of the vehicle.  Although this presently represents a second tether, the link could be established in the future by a radio link. The Trike’s stereo vision system is composed of two “off-the-shelf” Logitech QuickCam Pro cameras which are used to track the Trike (see Figure 4).  They are separated with a baseline of 1 meter, and are fixed in place so they are as coplanar as possible.  Each camera was calibrated independently for the internal parameters with the use of the freely available Intel® Camera Calibration Software.  The Trike is augmented with a large visual target which can be easily segmented from the background, and the position of the vehicle estimated.

The vehicle is tracked by colouring the visual target distinctively. Standard color tracking techniques (described in [11]) are used to segment the stereo images obtained by the cameras into regions of high probability that contain the target.  Instead of segmenting the images in RGB space, segmentation is based on the hue value of individual pixels (see Figure 5).  Segmentation is approached as a pixel classification process based on Bayesian probabilities. Conditional probabilities are used to determine the probability that a given pixel (hue) value corresponds to the visual target on the Trike.

Samples of images of the Trike in its operating environment are collected and conditional probabilities P(Hue|Object)and P(Hue|non−Object)are constructed as:

P(Hue|Object)=object[Hue]Tobj

P(Hue|non−Object)=non−object[Hue]Tnon−obj

In the above, Tobj and Tnon-obj are the total number of elements in the corresponding Object or Non-object histograms obtained from the sample images.  Here object[Hue] and non−object[Hue]are the bin counts of the corresponding histogram.  Given a particular Hue value, it is then possible to compute the conditional probability that the hue value corresponds to the object using Baye’s rule:

P(object|Hue)=P(Hue|object)P(object)P(Hue|Object)P(object)+P(Hue|non−object)P(non−object)

where:

P(object)=TobjTobj+Tnonobj and P(non−object)=TnonobjTobj+Tnonobj.

Once the probability of a pixel being part of the object has been calculated, pixels are classified as being part of the visual target as P(object|Hue)≥Δ, where Δis a predefined threshold.  Given the classification of pixels in a given image, the zero’th and first order moments are computed to identify the centroid of the largest region of classified pixels.

The visual target is tracked independently in both images, and an epipolar constraint is applied to ensure that the same target is identified in both images.  This is accomplished by calculating the tracked center of the object and checking to see if the left and right centers are within a given distance threshold both vertically and horizontally.  If the tracking regions deviate past these thresholds, the left image enforces the epipolar constraint on the right image by setting the search region to be along the same horizontal line in the image.  When the target is acquired once again, the constraint is lifted and the independent tracking resumes. Figure 6 shows the Trike being tracked by the visual system. The large red squares indicate the tracked target, with the center of the target marked with a large dot.

Between subsequent frames, the search region is moved in each image by means of a predictive Alpha-Beta Filter [12] which also helps to smooth the final data.  The update equation of the Alpha-Beta Filter is defined as:

x(k+1|k+1)=x(k+1|k)+αβΔtz(k+1)−z(k+1|k)

whereαand β are optimally defined in [12].

From frame to frame the location of the search region is updated as:

x(k+1)=x(k)+v(k)Δt

where x(k) is the current state vector (u,v image coordinates) and v(k) is the velocity of each component of the state.  The prediction allows us to follow the object with higher accuracy and to avoid getting confused with other objects that may have a similar hue.

The tracking data is sent to the Trike through a peer-to-peer network connection.  The Trike is thus able to use the data as it moves around the physical world.  At present, the model and tracking data are not integrated using a strategy such as Kalman filtering [13], rather the Trike operates in one of two modes:

- Internal Mode. The internal sensors of the Trike are used only

- Tracker Mode. The external video tracker is used.

A Kalman filter-based approach to merging the data in an ongoing manner is currently under development.

Figure 7 illustrates the effectiveness (and need) for a tracking system which makes reference to the external world.  Relying only on internal tracking leads to substantial drift (Fig 7b). The external visual tracking keeps the location of the Trike constrained to its true physical position (Fig 7c).

(a) Position of known landmarks in the environment

(b) Internal bike model position

(c) External sensor model position

Figure 7. The Trike is ridden in a tight circle through the landmarks A-F marked on the floor (a). b shows the position of the Trike measured by the internal sensors. The numbered points are shown only to explain the direction the Trike moves through the world. (c) shows the raw tracker data from the stereo tracker. Again, the tracks are numbered to indicate the direction in which the Trike moves. Some gating of the tracking data is still required – note the large error to the left hand side of (c) when the tracking data is temporarily lost.  (Note that the location of the numbers are not correlated between (b) and (c).)

# 5. Discussion

During self-motion we receive a number of different cues from a variety of sensory systems. These cues include: optic flow; auditory motion; vestibularly sensed angular and linear acceleration; proprioceptive and somatosensory information about the position and movements of the limbs; and knowledge that instructions have been sent to the various muscles. Each sensory system has different response properties and information from these sensory systems interact in complex and subtle ways to generate our perception of self-motion and orientation in the world.  It might be thought that virtual reality primarily seeks to mislead the visual sense. In fact, the process of providing an imaginary space that a person w is able to explore, seeks to fool the totality of the various systems that contribute to spatial awareness.

Virtual Reality systems, which augment or modify one or more of these sensory systems, may confound our overall sensation of motion in various ways.  A common result of this confusion is nausea (cyber-sickness) and an associateddegradation of performance. Designers of immersive visual systems must take great care that their augmentation of vision or other sensory inputs does not interfere unpredictably with the normal perceptual processes.  Trike utilizes standard virtual reality technologies to generate an immersive visual display while utilizing real motion to generate compatible non-visual cues.  This permits operators a wider operational range than is found in more traditional virtual reality systems. Limited only by the length of the power cord and the available free space, subjects can explore large virtual environments and obtain appropriate visual and non-visual cues to their motion.

# Bibliography

[1]	Matias, E., MacKenzie, I. S., and Buxton, W., A wearable computer for use in microgravity space and other non-desktop environments, Proc. Companion of thee CHI’96 Conference on Human Factors in Computing Systems, pp. 69-70, New York, NY, 1996.

[2]	Ockerman, J. J. and Pritchett, A., R., Preliminary investigation of wearable computers for task guidance in aircraft inspection, Proc. III 2nd Int. Symp. on Wearable Computers, Pittsburgh, PA, 1998.

[3]	Smailagic, A., and Siewiorek, D., User-centred interdisciplinary concurrent system design, IBM Systems Journal, 1999.

[4]	Lewis, S. A., Havey, G. D., and Hanzal, B., Handheld and bodyworn graphical displays, Proc. IEEE 2nd Int. Symp. on Wearable Computers, Pittsburgh, PA, 1998.

[5]	Feiner, S., McIntyre, B., Höllerer, T., and Webster, A., A touring machine: prototyping 3d mobile augmented realityh systems for exploring the urban environment, Proc. IEEE 1st Int. Symp. on Wearable Computers, Cambridge, MA, 1997.

[6]	Dudek, G., and Jenkin, M. Computational Principles of Mobile Robotics, Cambridge University Press, New York, NY, 2000.

[7]	Bülthoff, H. and van Veen, H. A. C.  Vision and action in Virtual environments. In M. Jenkin and L. Harris (Eds.) Vision and Attention, Springer Verlag: New York, NY, 1999.

[8]	Gradecki, J. The Virtual Reality Construction Kit, John Wiley and Sons, New York, NY, 1994.

[9]	Iwata, H. Walking about virtual environments on infinite floor. Proc. IEEE Int. Symp. On Virtual Reality, 1999.

[10]	Darken, R. P., Cockayne, W. R., and Carmein, D. (1997). The Omni-Directional Treadmill: A locomotion device for virtual world, Proc. UIST’97, pp. 213-221.

[11]	Jones, M. J.,  and Rehg, J. M.  Statistical color models with applications to skin detection.  Technical Report CRL 98/11, Compaq Computer Corp., Cambridge, MA USA, 1998.

[12]	Brown, C. (Editor).  Tutorial on Filtering, Resotration, and State Estimation.  Technical Report 534.  Dept. Computer Science.  University of Rochestor.  Rochester, NY. 1994.

[13]	Bozic, S. M. Digital and Kalman Filtering, Edward Arnold, 1979.

# Acknowledgments

Financial support from the Centre for Research in Earth and Space

Technology (CRESTech) and NSERC Canada are greatfully acknowledged.